*******************************************************************************

EBSTAT

*******************************************************************************

CUPL            3.2a Serial# MD-32A-6918

Device          p22v10  Library DLIB-h-25-17

Created         Mon Jun 04 12:56:47 1990

Name            EBSTAT

Partno          EV80C186EB

Revision        01

Date            06/04/90

Designer        Garry Mion

Company         Intel ECO

Assembly        80C186EB Evaluation Board

Location        U16

===============================================================================

Expanded Product Terms

===============================================================================

510_cs =>

gcs6 & !la4 & !la5 & !resout

config =>

gcs6 & la4 & !la5 & rd & !resout

cpu_fetch =>

ls0 & ls1 & !ls2

cpu_halt =>

!ls0 & !ls1 & ls2

cpu_inta =>

ls0 & ls1 & ls2

cpu_passive =>

!ls0 & !ls1 & !ls2

fetch =>

ls0 & ls1 & !ls2 & !resout

halt =>

!ls0 & !ls1 & ls2 & !resout

inta =>

ls0 & ls1 & ls2 & !resout

io_read =>

!ls0 & ls1 & ls2

io_write =>

ls0 & !ls1 & ls2

iord =>

!ls0 & ls1 & ls2 & !resout

iowr =>

ls0 & !ls1 & ls2 & !resout

memory_read =>

!ls0 & ls1 & !ls2

memory_write =>

ls0 & !ls1 & !ls2

memr =>

!ls0 & ls1 & !ls2 & !resout

memw =>

ls0 & !ls1 & !ls2 & !resout

rec =>

dt_r

510_cs.oe  =>

1

config.oe  =>

1

fetch.oe  =>

1

halt.oe  =>

1

inta.oe  =>

1

iord.oe  =>

1

iowr.oe  =>

1

memr.oe  =>

1

memw.oe  =>

1

rec.oe  =>

1

===============================================================================

Symbol Table

===============================================================================

Pin Variable                                    Pterms   Max     Min

Pol   Name              Ext     Pin     Type     Used   Pterms  Level

--- --------            ---     ---     ----    ------  ------  -----

!  510_cs                      14       V        1       8       3

ale                         8        V        -       -       -

!  config                      15       V        1       10      3

cpu_fetch                   0        I        1       -       -

cpu_halt                    0        I        1       -       -

cpu_inta                    0        I        1       -       -

cpu_passive                 0        I        1       -       -

dt_r                        13       V        -       -       -

!  fetch                       19       V        1       16      3

!  gcs6                        7        V        -       -       -

!  halt                        20       V        1       14      3

!  inta                        23       V        1       8       3

io_read                     0        I        1       -       -

io_write                    0        I        1       -       -

!  iord                        22       V        1       10      3

!  iowr                        21       V        1       12      3

la4                         10       V        -       -       -

la5                         11       V        -       -       -

!  ls0                         2        V        -       -       -

!  ls1                         3        V        -       -       -

!  ls2                         4        V        -       -       -

memory_read                 0        I        1       -       -

memory_write                0        I        1       -       -

!  memr                        18       V        1       16      3

!  memw                        17       V        1       14      3

!  rd                          6        V        -       -       -

!  rec                         16       V        1       12      3

resout                      9        V        -       -       -

!  wr                          5        V        -       -       -

510_cs              oe      14       D        1       1       0

config              oe      15       D        1       1       0

fetch               oe      19       D        1       1       0

halt                oe      20       D        1       1       0

inta                oe      23       D        1       1       0

iord                oe      22       D        1       1       0

iowr                oe      21       D        1       1       0

memr                oe      18       D        1       1       0

memw                oe      17       D        1       1       0

rec                 oe      16       D        1       1       0

LEGEND    F : field      D : default variable        M : extended node

N : node       I : intermediate variable   T : function

V : variable   X : extended variable       U : undefined